❗ Problem Statement APTech SlimBytes 2026 — Why existing memory management is broken on ARM64

⚠ THE MEMORY ALLOCATION TRAP App requests 256 MB of memory ▼ OPTION A ← Today's default 4 KB pages 65,536; Page Table Entries. 512 KB; Wasted as page tables. 16×; More TLB misses per 64 KB region. ❌ Performance degrades; OPTION B ← Classic THP 2 MB THP pages 128; PTEs (good!). 1.8 MB; Internal fragmentation bloat. 100%; All apps get same size — no fit. ❌ Memory bloat explodes. ⚡ Neither approach solves the full-system picture — a smart, adaptive, per-process solution is needed.

🔍 4 Root Causes — Why This Problem Exists

| 01 |  | 📄 Page Table Memory Explosion at 4 KB Mapping 1 GB of workload with 4 KB pages requires 262,144 Page Table Entries consuming 2 MB of RAM — purely as overhead. On memory-constrained WearOS/mobile devices with 2–4 GB RAM, this overhead is visible and measurable in /proc/meminfo. No existing kernel mechanism automatically reduces this. |  | KEY METRIC 2 MB overhead |
| --- | --- | --- | --- | --- |

| 02 |  | 💥 2 MB THP Causes Severe Internal Fragmentation Classic Transparent Huge Pages force 2 MB allocations system-wide regardless of actual buffer size. A 256 KB ML tensor allocated as 2 MB wastes 1.75 MB per allocation. GPU/camera buffers that are not 2 MB-aligned cause page cache pollution and TLB shootdowns that undo all THP benefits. |  | KEY METRIC 1.8 MB bloat/alloc |
| --- | --- | --- | --- | --- |

| 03 |  | 🔧 Per-Driver Manual Fixes Do Not Scale Current solutions (e.g. KGSL Vendor Hook mTHP) require a kernel engineer to hand-implement shmem_suitable_orders() in every driver — GPU today, camera HAL tomorrow, codec the day after. Each subsystem is a separate patch. New apps or new Qualcomm IPs get no benefit until someone writes the code. This approach does not scale across the Android/LE software stack. |  | KEY METRIC N patches for N drivers |
| --- | --- | --- | --- | --- |

| 04 |  | 📊 No System-Wide Measurement or Feedback Loop Linux provides /proc/vmstat counters (thp_fault_alloc, thp_split_page) but no tool correlates THP promotion decisions with per-process memory savings in real time. Fragmentation state (buddy allocator free pages) is not fed back into the THP selection path — so the kernel can promote pages into a nearly-full buddy zone, causing immediate splits and negating all gains. |  | KEY METRIC No live feedback loop |
| --- | --- | --- | --- | --- |

| 💡 Gap Summary — What ARM64 Devices Are Missing |  |  |  |
| --- | --- | --- | --- |
| 🎯 Per-process page size selection Kernel treats all apps identically | 🔁 Real-time buddy feedback No fragmentation guard in THP path | 📐 Zero driver changes needed Every subsystem needs manual patch | 📉 Quantified RAM savings No tool measures MB freed per app |

arm64-mTHP Autopilot solves all 4 root causes simultaneously — zero driver changes, system-wide, with 1.9 MB of real RAM freed and measurable in /proc/meminfo.