Problem Statement Why memory management on ARM64 is broken today

The Memory Allocation Trap App requests 256 MB → kernel must choose a page size → both options fail

Option A · 4 KB Pages Today's default across all Linux kernels 65,536; Page Table Entries needed. 512 KB; RAM wasted as page tables. 16 ×; More TLB misses per 64 KB. Slow · High TLB pressure · Bad for ML & games; Option B · 2 MB THP Pages Classic Transparent Huge Pages, system-wide 128; PTEs (looks good!). 1.8 MB; Internal fragmentation each. 100 %; Apps forced same size — no fit. Memory bloat · Cache pollution · Inflexible. Neither option fits a real workload. A per-process adaptive page size is the only solution.

4 Root Causes — Why This Problem Exists

| 01 |  | 📄 Page tables eat real RAM Every 4 KB page needs its own entry in the page table. Map 1 GB of memory and you need 2 MB just for bookkeeping — RAM that could have been free for your app. |  | KEY METRIC 2 MB overhead per GB mapped |
| --- | --- | --- | --- | --- |

| 02 |  | 💥 2 MB pages waste more than they save Forcing 2 MB huge pages on a 300 KB ML tensor wastes 1.7 MB per allocation. GPU and camera buffers that don't align to 2 MB boundaries cause fragmentation that is worse than using 4 KB pages. |  | KEY METRIC 1.8 MB wasted per mis-aligned alloc |
| --- | --- | --- | --- | --- |

| 03 |  | 🔧 Every driver needs a separate hand-written fix Current solutions require a kernel engineer to patch each driver individually — GPU today, camera HAL tomorrow, codec next week. New chips and new apps get no benefit until someone writes the code. This will never scale. |  | KEY METRIC 1 patch per driver — endless work |
| --- | --- | --- | --- | --- |

| 04 |  | 📊 No feedback loop between allocator and decision The kernel promotes pages to huge size without checking whether free huge pages are actually available. Under fragmentation it promotes, fails, and splits — causing more overhead than doing nothing. There is no live feedback channel. |  | KEY METRIC Blind promotion causes THP splits |
| --- | --- | --- | --- | --- |

arm64-mTHP Autopilot addresses all 4 root causes in one shot Zero driver changes · Works on every app automatically · 1.9 MB of real RAM freed